Digital Egypt Pioneers Initiative – DEPI

                            Advanced FortiGate Security Profiles




Prepared by:

1- Ebrahim Sabry Farouk

2-Hussein Hisham Hussein

3- Ahmed Samy Abdallah Shokr

4-Mohamed Rabia Fathi

5-Ahmed Maged Mohamed

6-Bishoy Remon Fahmy

Instructor Name : Eng. El Hosein Ahmed

## 1. Project Overview

## This project represents the design and implementation of a complete secured network infrastructure for a company. The objective is to build a fully functional multi‑VLAN network with redundancy, security, dynamic routing, and Layer‑3 switching following enterprise standards.



2. Network Design Summary

The design uses Cisco-based devices including Core switches, Distribution switches (DSW‑1, DSW‑2), and Access switches for each department. The network follows a hierarchical model:
- Core Layer
- Distribution Layer (DSW‑1 & DSW‑2)
- Access Layer (Finance, Marketing, Sales, HR, IT)

on HQ-Branch

## 3. VLAN Structure

The network utilizes five main VLANs:
• VLAN 10 – Finance – 192.168.1.0/24
• VLAN 20 – Marketing – 192.168.2.0/24
• VLAN 30 – Sales – 192.168.3.0/24
• VLAN 40 – HR – 192.168.4.0/24
• VLAN 50 – IT – 192.168.5.0/24

## . SVI Configuration (DSW‑1 & DSW‑2)

Each Distribution Switch contains an SVI for every VLAN:
Example (DSW‑1):
VLAN 10 – 192.168.1.254/24
VLAN 20 – 192.168.2.254/24
VLAN 30 – 192.168.3.254/24
VLAN 40 – 192.168.4.254/24
VLAN 50 – 192.168.5.254/24

DSW‑2 uses the same VLANs but with .253 as gateway backup.



On Sec-Branch
The network utilizes five main VLANs:

VLAN 10– IT – 10.1.1.0/24
VLAN 20 –HR– 20.1.1.0/24
VLAN 30–Marketing – 30.1.1.0/24
VLAN 40–Sales –40.1.1.0/24
VLAN 50 –Finance– 50.1.1.0/24

. SVI Configuration (DSW‑1 & DSW‑2)

Each Distribution Switch contains an SVI for every VLAN:

Example (DSW‑1):
VLAN 10–– 10.1.1.254/24
VLAN 20 –– 20.1.1.254/24
VLAN 30–– 30.1.1.254/24
VLAN 40––40.1.1.254/24
VLAN 50 –– 50.1.1.254/24

DSW‑2 uses the same VLANs but with .253 as gateway backup.

## 5. VTP Configuration
On HQ-Branch :

VTP Domain: cisco.com
Password: 123
DSW‑1: Server mode
DSW‑2: Server mode
Access Switches: Client mode

Spanning Tree Configuration (STP) :

DSW-1:

Vlan 10,20,30 priority 4096   active for VLANs 10,20,30

Vlan 40,50 priority 8192  backup for VLANs 40,50

DSW-2 :

Vlan 40,50  priority 4096 active for VLANs 40,50

Vlan 10,20,30 priority 8192    backup for VLANs 10,20,30

On Sec-Branch :
VTP Domain: naruto.com
Password: 123
DSW-1: Server mode
DSW-2: Server mode
Access Switches: Client mode

Spanning Tree Configuration (STP) :

On HQ-Branch :

DSW-1:

Vlan 10,20 priority 4096   active for VLANs 10,20

Vlan 30,40,50 priority 8192  backup for VLANs 30,40,50

DSW-2 :
 Vlan 30,40,50 priority 4096 active for VLANs 30,40,50

Vlan 10,20 priority 8192    backup for VLANs 10,20







On Sec-Branch :

DSW-1:

Vlan 10,20,30 priority 4096   active for VLANs 10,20,30

Vlan 40,50 priority 8192  backup for VLANs 40,50

DSW-2 :
 Vlan 40,50 priority 4096 active for VLANs 40,50

Vlan 10,20,30 priority 8192  backup for VLANs 10,20,30

## 6. Trunking & DTP

All uplinks are configured as 802.1Q trunks, allowing VLANs 10,20,30,40,50 with native VLAN 100.
Distribution switches use Dynamic Desirable while Access switches use Dynamic Auto.

## 7. Port Security

Applied on Access Switches for user ports:
- Maximum 2 MAC addresses
- Sticky MAC enabled
- Violation mode: Restrict
- PortFast enabled on access ports

## 8. Redundancy (HSRP)

HSRP provides gateway redundancy:
- Virtual gateway for each VLAN (192.168.x.100)
On HQ-Branch : 
- DSW‑1
  active for VLANs 10, 20    priority 110
  Standby for VLANs 30,40, 50  priority 90
- DSW‑2 
  active for VLANs 30,40,50    priority 110
  Standby for VLANs 10,20  priority 90
On Sec-Branch :
- DSW‑1
  active for VLANs 10, 20,30    priority 110
  Standby for VLANs 40, 50  priority 90
- DSW‑2 
  active for VLANs 40,50    priority 110
  Standby for VLANs 10,20,30  priority 90

## 9. EtherChannel

EtherChannel (Port‑channel 10) connects DSW‑1 and DSW‑2 using Layer‑3 links:
 DSW‑1: 11.1.1.1/30
 DSW‑2: 11.1.1.2/30
 Mode: ON

## 10. Routing (OSPF)

All Distribution and Core switches run OSPF Area 0.
SVIs are passive interfaces.
OSPF networks include all VLANs and inter‑switch links.

## 11. Security Measures Implemented

• VLAN Segmentation
• HSRP Redundancy
• STP Loop Prevention
• Port Security
• VTP Authentication
• Dynamic Trunking Control

## 12. Conclusion

The network is fully operational, redundant, secure, and scalable. It follows best practices of the Cisco Enterprise Architecture and is ready for further expansion or integration with firewalls such as FortiGate in later phases.

FortiGate Section :

1. Interface Configuration

Three main interfaces were configured on the FortiGate firewall to separate traffic and ensure proper network segmentation:

- LAN Interface:
Assigned an IP address and connected to the internal core switch to handle all internal VLAN traffic.

- WAN Interface:
Configured with an IP address to provide internet access for the whole network.

- Management Interface:
Dedicated interface used only for administrative access and secure remote management of the firewall.

2. Dynamic VLAN Learning Through OSPF

Instead of manually creating VLANs on the FortiGate, OSPF was enabled so the firewall can dynamically learn all internal VLAN networks from the Core Switch.

- OSPF was activated in the firewall.

- The FortiGate successfully learned all VLAN subnets advertised by the core.

- This ensures easier scalability and reduces configuration overhead.

3. Firewall Policies

Several policies were created to secure and control traffic flow across the network:

a. LAN → WAN Policy

Allows internal users to access the internet.
NAT was enabled to translate internal IP addresses to public IP.

b. WAN → LAN Policy

Allows inbound traffic from the internet to reach internal servers when required.
Used for services published through the firewall.

4. Security Profiles Applied to Policies

To enhance security, multiple FortiGate security features were applied:

a. Antivirus Profile

Attached to inbound traffic (WAN → LAN) to prevent any malicious files or viruses from entering the network.

b. Web Filter Profile

Applied on outbound traffic (LAN → WAN) to control internet usage, block unethical content, and restrict access to non-business websites.

c. Application Control

Used to block social media and non-productive applications.
Ensures users stay focused on company-related tasks.

5. IPsec VPN Between Branches

A site-to-site IPsec VPN tunnel was created to connect the branches securely:

- Phase 1 and Phase 2 configurations completed.

- Tunnel established successfully.

- Routing was added to ensure seamless communication between both sides.

- A specific VPN policy was created to allow traffic through the tunnel.

6. Routing Configuration

Routing was implemented as follows:

- Default Route:
A 0.0.0.0/0 route pointing to the WAN gateway was added to forward all internet-bound traffic.

- Inter-Branch Routing:
Static or OSPF routes were configured so both branches can reach each other's internal networks through the IPsec tunnel.

7. High Availability (HA) Configuration

To ensure business continuity:

- Primary (Active) Unit and Secondary (Passive) Unit were configured in HA mode.

- If the primary device fails, the secondary FortiGate automatically takes over.

- Provides redundancy and prevents downtime.

8. Overall Security & Network Integration

The FortiGate firewall was integrated smoothly into the existing CCNA-designed network:

- Learns VLANs via OSPF

- Secures traffic with advanced security profiles

- Provides controlled internet access

- Connects branches via VPN

- Ensures uptime through HA

- Integrates with the core switch running OSPF and VLAN segmentation

This setup delivers a complete enterprise-grade network combining both Cisco (CCNA Layer-2/Layer-3) and FortiGate security solutions.